The University of Azad Jammu and Kashmir

Name : Nida Hameed

Session : 2024-2028

Course Title: CA&LD

Submitted to : Engr . Sidra Rafique

De Morgan’s Law for NAND Gate:

Procedure

I opened Electronics Workbench and started a new project.

- I added the 7400 IC (NAND gate), 7404 IC (NOT gate), and 7432 IC (OR gate) to the workspace.

- I connected Vcc (pin 14) and GND (pin 7) on all three ICs.

- For the left side of the circuit, I used a single NAND gate from the 7400:

  - I connected two switches to the inputs (pins 1 and 2).

  - The output was taken from pin 3, which showed the direct NAND result of A and B.

- For the right side, I built the equivalent circuit using De Morgan’s Law:

  - I took the same two inputs (A and B) and connected each to a NOT gate .

  - Their outputs were then connected to an OR gate .

  - This way I created: A' + B' which, according to De  Morgan’s Law, is equal to (A ⋅ B)’.

- I connected LEDs to both outputs (NAND and OR-NOT combo) to compare them.

- I tested all input combinations using the two switches:

  - (0,0), (0,1), (1,0), (1,1)

which is De Morgan’s Theorem.

- The LEDs lit up the same on both sides every time, so the proof worked perfectly.

- This was a clean and clear way to verify De Morgan's Law using logic gates and ICs.

Formula:

(A.B)’=A’+B’

Hardware circuit

Table:

| A | B | A’ | B’ | A.B | (A.B)’ | A’+B’ |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 0 | 0 | 1 | 0 | 0 |
| 1 | 0 | 0 | 1 | 0 | 1 | 1 |
| 0 | 1 | 1 | 0 | 0 | 1 | 1 |
| 0 | 0 | 1 | 1 | 0 | 1 | 1 |

De Morgan’s law for NOR Gate:

Procedure

I opened Electronics Workbench and started a new project.

- I added the 7402 IC (NOR gate), 7408 IC (AND gate), and 7404 IC (NOT gate) to the workspace.

- I connected Vcc (pin 14) and GND (pin 7) on all three ICs.

- On the left side of the circuit, I used a single NOR gate :

  - I connected two switches as inputs to pins 1 and 2 of the IC.

  - The output from pin 3 gave me NOR(A, B) directly.

- On the right side, I built the De Morgan equivalent using NOT and AND gates:

  - I passed the same two inputs through two NOT gates from the 7404.

  - Then I connected those inverted outputs to an AND gate from the 7408.

  - This built: A' ⋅ B' which is the same as (A + B)’ by De Morgan’s Law.

- I connected LEDs to both outputs to visually compare the NOR and the AND-NOT combination.

- I tested all input combinations using the switches:

  - (0,0), (0,1), (1,0), (1,1)

- For every test, both outputs matched showing the same logic level on both LEDs.

- The circuit worked exactly how it should  both sides gave the same output for all input cases.

Formula:

(A+B)’=A’.B’

Table:

| A | B | A’ | B’ | A+B | (A+B)’ | A’.B’ |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 0 | 0 | 1 | 0 | 0 |
| 1 | 0 | 0 | 1 | 1 | 0 | 0 |
| 0 | 1 | 1 | 0 | 1 | 0 | 0 |
| 0 | 0 | 1 | 1 | 0 | 1 | 1 |