The University of Azad Jammu and Kashmir

Name : Nida Hameed

Session : 2024-2028

Course Title: CA&LD

Submitted to : Engr . Sidra Rafique

Half Adder

Half Adder simple circuit

Procedure: How I Created a Simple Half Adder Circuit

- Objective:
I wanted to make a half adder that adds two 1-bit inputs (A and B) and gives me the sum and carry outputs using a simple circuit.

- Understanding the Half Adder:
I know the half adder takes inputs A and B and produces:

- Sum = A XOR B

- Carry = A AND B

Since this is a simple circuit, I directly used XOR and AND gates for these outputs.

- Building the Circuit:

- I connected inputs A and B directly to an XOR gate to get the Sum output.

- Then I connected inputs A and B to an AND gate to get the Carry output.

- Testing the Circuit:
I tested all possible inputs:

- A=0, B=0 → Sum=0, Carry=0

- A=0, B=1 → Sum=1, Carry=0

- A=1, B=0 → Sum=1, Carry=0

- A=1, B=1 → Sum=0, Carry=1

The outputs were exactly what I expected.

- Conclusion:
By using the XOR and AND gates directly, I built a simple and efficient half adder. This helped me understand how these basic gates work together to perform addition.

Hardware circuit:

Half Adder Complicated circuit

Procedure: How I Created a Half Adder Using NOT, AND, and OR Gates

- Objective:
I wanted to build a half adder that adds two inputs and outputs sum and carry, but only using NOT, AND, and OR gates because XOR gate wasn’t allowed.

- Understanding the Logic:
I know a half adder has two inputs (A and B) and two outputs:

- Sum = A XOR B

- Carry = A AND B

Since I can’t use XOR directly, I needed to express XOR using only NOT, AND, and OR gates.

3.Building the Circuit:

- I connected input A to a NOT gate to get A’, and input B to AND gate.

- Then I connected input A with AND gate and input B with NOT gate.

- Then I ANDed A’ with B, and A with B’.

- I ORed the results of those two AND gates to produce the sum output.

- For carry, I simply ANDed A and B directly.

4. Testing the Circuit:
I tested the circuit for all possible inputs:

- For A=0, B=0 → Sum=0, Carry=0

- For A=0, B=1 → Sum=1, Carry=0

- For A=1, B=0 → Sum=1, Carry=0

- For A=1, B=1 → Sum=0, Carry=1

Everything worked exactly as expected.

5.Conclusion:
By building the half adder using NOT, AND, and OR gates, I learned how to create XOR functionality from basic gates. This helped me understand how complex logic circuits are made from simple parts.

Hardware circuit:

Table:

| A | B | A’ | B’ | A’.B | A.B’ | A’.B+A.B’(Sum) | A.B(carry) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 0 | 0 | 0 | 0 | 0 | 1 |
| 1 | 0 | 0 | 1 | 0 | 1 | 1 | 0 |
| 0 | 1 | 1 | 0 | 1 | 0 | 1 | 0 |
| 0 | 0 | 1 | 1 | 0 | 0 | 0 | 0 |